APPENDIX A

ACHIEVEMENT TEST ON ELECTRONIC CIRCUITS AND ITS APPLICATIONS

FOR ELECTRONICS ENGINEERING TECHNOLOGY STUDENTS IN POLYTECHNICS IN RIVERS STATE

Name of Polytechnic (Optional): --------------------------------------------------------------------

Level: ---------------------------------------------------------------------------------------------------

Session: ------------------------------------------------------------------------------------------------

Marks: 60

Time: 2hours

GENERAL INSTRUCTION

Do not open this paper until you are told to do so.

This paper consists of 60 objective questions, with five sections. Each section consists of questions drawn from the following areas: amplifiers, rectifiers, portable USB charger, Infrared Proximity sensor, and Simple Touch Sensor. Choose from letter (A-D) that corresponds with the correct answer. All questions carry equal marks

Multiple Choice Questions on Amplifiers

1.	An amplifier operating from ±3V provide a 2.2V peak sine wave across a 100 ohm load when provided with a 0.2V peak sine wave as an input from which 1.0mA current is drawn. The average current in each supply is measured to be 20mA. What is the amplifier efficiency?      (a) 20.2%   (b) 25.2%   (c) 30.2%   (d) 35.2%

2.	In order to prevent distortion in the output signal after amplification, the input signal must be.  (a) Higher than the positive saturation level of the amplifier (b) Lower than the negative saturation level of the amplifier (c) Must lie with the negative and the positive saturation level of the amplifier (d) Both higher than the positive saturation level of the amplifier and lower than the negative saturation level of the amplifier.

3.	The voltage gain of the amplifier is 8 and the current gain is 7. The power gain of the amplifier is (a) 56 db (b) 17.481 db (c) 34.963 db (d) 1 db.

4.	Statement 1: Voltage gain of -5 means that the output voltage has been attenuated.

Statement 2: Voltage gain of -5db means that the output voltage has been attenuated. (a) Statement 1 and Statement 2 are true (b) Statement 1 and Statement 2 are false (c) Only Statement 1 is true (d) Only Statement 2 is true.

5. 	Which of the following isn’t true? (a) Both transformer and amplifier can provide voltage gain (b) Both transformer and amplifier can provide current gain (c) Both transformer and amplifier can provide power gain (d) None of the mentioned

6. 	Symmetrically saturated amplifiers operating in clipping mode can be used to convert a sine wave to a (a) Square wave (b) Pseudo Square wave (c) Saw tooth wave (d) Triangular wave

7. 	What is meant by stability of an amplified signal? (a) The amplified signal must have finite amplitude (b) The amplified signal should not have self-oscillation (c) The input and the output signal must be proportional (d) The ratio of the input and the output signal must be finite

8.	 If Av, Ai and Ap represents the voltage gain, current gain and power gain ratio of an amplifier which of the below is not the correct expression for the corresponding values in decibel? (a) Current gain: 20 log Ai db (b) Voltage gain: 20 log Av db (c) Power gain: 20 log Ap db (d) Power gain: 10 log Ap

9. 	An amplifier has a voltage gain of 100 V/V and a current gain of 1000A/A. the value of the power gain decibel is (a) 30 db (b) 40 db (c) 50 db (d) 60 db

10. 	The units of voltage gain is (a) It has no units, it is a ratio (b) Decibels (db) (c) All of the mentioned (d) None of the mentioned

Multiple Choice Questions on Rectifiers

11. 	Which of the following isn’t a type of rectifier? (a) Precision Half-wave Rectifier (b) Bridge Rectifier (c) Peak Rectifier (d) None of the mentioned

12. 	For a half wave or full wave rectifier the Peak Inverse Voltage of the rectifier is always (a) Greater than the input voltage (b) Smaller than the input voltage (c) Equal to the input voltage (d) Greater than the input voltage for full wave rectifier and smaller for the half wave rectifier

13. 	For a half-wave rectifier having diode voltage VD and supply input of VI, the diode conducts for π – 2Θ, where Θ is given by (a) tan -1 VD/VI (b) tan-1 VD/VI – VI (c) sin-1 VD/VI (d) sin-1 VD/VI – VI

14. 	Bridge rectifier is an alternative for (a) Full wave rectifier (b) Peak rectifier (c) Half wave rectifier (d) None of the mentioned

15. 	Which of the following is true for a bridge rectifier? (a) The peak inverse voltage or PIV for the bridge rectifier is lower when compared to an identical center tapped rectifier (b) The output voltage for the center tapped rectifier is lower than the identical bridge rectifier (c) A transistor of higher number of coil is required for center tapped rectifier than the identical bridge rectifier (d) All of the mentioned

16. 	The diode rectifier works well enough if the supply voltage is much than greater than 0.7V. For smaller voltage (of few hundreds of millivolt) input which of the following can be used? (a) Super diode (b) Peak rectifier (c) Precision rectifier (d) None of the mentioned

17. 	A simple diode rectifier has ‘ripples’ in the output wave which makes it unsuitable as a DC source. To overcome this one can use (a) A capacitor in series with a the load resistance (b) A capacitor in parallel to the load resistance (c) Both of the mentioned situations will work (d) None of the mentioned situations will work

18. 	Consider a peak rectifier fed by a 60-Hz sinusoid having a peak value Vp = 100 V. Let the load resistance R = 10 kΩ. Calculate the fraction of the cycle during which the diode is conducting (a) 1.06 % (b) 2.12 % (c) 3.18% (d) 4.24%

(Q.19-Q.20) The op amp in the precision rectifier circuit is ideal with output saturation levels of	±12 V. Assume that when conducting the diode exhibits a constant voltage drop of 0.7 V.

19. Find V– when VI is -1V. (a) 0V (b) 0.7V (c) 1V (d) 1.7V

20. 	Find V0 when VI is 2V. (a) 0V (b) 0.7V (c) 1V (d) 1.7V

Appendix B

Electrical/Electronics Technology Option. Department of Industrial Technical Education,

Faculty of Vocational Technical Education

University of Nigeria, Nsukka.

30th Sept, 2019.

Dear Respondents,

INTEREST INVENTORY ON EFFECTS OF PRACTICAL INSTRUCTIONAL GUIDE AND TEACHING NAKED TECHNIQUES ON STUDENTS ACHIEVEMENT, INTEREST AND RETENTION IN ELECTRONICS ENGINEERING TECHNOLOGY IN POLYTECHNICS IN RIVERS STATE

We are students of the above mentioned department. I am conducting a research on Effects of Practical Instructional Guide and Teaching Naked Technique on Students Achievement, Interest and Retention in Electronics Engineering Technology in Polytechnics in Rivers State. Please kindly respond to the items on the interest inventory, as you feel appropriate. Information supplied will be strictly confidential and used for the purpose of this study only.

Thanks for your co-operation.

Yours Sincerely,

Abulokwe, Amaechi Clement & Youdiowei Budouaki Terry

(Researchers)

INTEREST INVENTORY

KEY:

SA - Strongly Agree; A – Agree; U- Undecided D - Disagree; and SD - Strongly Disagree;

Instruction: Check (√) any of the five response options according to your agreement or disagreement to the following statements.

| S/N | Items | SA | A | U | D | SD |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | The materials used in teaching each topic are organized |  |  |  |  |  |
| 2 | Helping other learners in original or imaginative ways, with autonomy and independence |  |  |  |  |  |
| 3 | The material was excellent as well as the activities. |  |  |  |  |  |
| 4 | I really liked this for my job skills training. |  |  |  |  |  |
| 5 | Looking forward to using this as part of our preparation for Career in electronics technology |  |  |  |  |  |
| 6 | I liked it because it was simple for me to complete |  |  |  |  |  |
| 7 | Easy to use and helped as we began our career research projects. |  |  |  |  |  |
| 8 | Identify career options consistent with your interests |  |  |  |  |  |
| 9 | Direct your own career exploration at various stages in your life |  |  |  |  |  |
| 10 | Prefers practical learning environments; learn by doing; |  |  |  |  |  |
| 11 | prefers short-term training to achieve a specific goal or skill |  |  |  |  |  |
| 12 | Prefers accomplishing tasks independently |  |  |  |  |  |
| 13 | Enjoys role as independent contributor |  |  |  |  |  |
| 14 | likes to solve problems on one’s own |  |  |  |  |  |
| 15 | Enjoy learning environments that allow you to be part of a team. |  |  |  |  |  |
| 16 | Prefer collaborating on shared team goals. |  |  |  |  |  |
| 17 | Like solving problems with others rather than by yourself |  |  |  |  |  |
| 18 | Knowledge of principles and methods for curriculum delivery, teaching and instruction for individuals and groups, and the measurement of training effects |  |  |  |  |  |
| 19 | Skill in teaching others how to do something |  |  |  |  |  |
| 20 | Skill in selecting and using instructional methods and procedures appropriate for the situation |  |  |  |  |  |

Appendix C

DEVELOPED PRACTICAL ASSESSMENT GUIDE ON ELECTRONIC CIRCUITS AND ITS APPLICATIONS FOR ELECTRONICS ENGINEERING TECHNOLOGY STUDENTS IN POLYTECHNICS IN RIVERS STATE

KEY:

SA - Strongly Agree; A – Agree; U- Undecided D - Disagree; and SD - Strongly Disagree;

Instruction: Check (√) any of the five response options according to your agreement or disagreement to the following statements.

| S/N | ITEMS | SA | A | U | D | SD |
| --- | --- | --- | --- | --- | --- | --- |
|  | CONSTRUCTION SKILL |  |  |  |  |  |
| 1 | Active components are correctly fixed on Vero board |  |  |  |  |  |
| 2 | Passive components are correctly fixed on Vero board |  |  |  |  |  |
| 3 | Soldering of parts are neat and bright |  |  |  |  |  |
| 4 | wires used are correctly fixed |  |  |  |  |  |
| 5 | Circuit is well grounded |  |  |  |  |  |
| 6 | Components layout on the Vero board are well spaced |  |  |  |  |  |
| 7 | Underside layout of Vero board is neat |  |  |  |  |  |
| 8 | Materials are well economized |  |  |  |  |  |
| 9 | Tools handled skillfully |  |  |  |  |  |
| 10 | Equipment operated skillfully |  |  |  |  |  |
| 11 | Student worked independently with little or no assistance from the teacher / instructor |  |  |  |  |  |
| 12 | Safety measures are strictly adhered to during construction |  |  |  |  |  |
| 13 | Project is completed within reasonable time |  |  |  |  |  |
|  | ASSEMBLING SKILLS |  |  |  |  |  |
| 14 | Component parts are properly fixed on the base board or plate |  |  |  |  |  |
| 15 | Base board and circuit are well grounded |  |  |  |  |  |
| 16 | Link wire between circuits and component parts are of correct length |  |  |  |  |  |
| 17 | External fittings are properly fixed |  |  |  |  |  |
| 18 | Joints are firm |  |  |  |  |  |
| 19 | Appropriate jack socket outlets are provided |  |  |  |  |  |
| 20 | Power supply cable is correctly fixed |  |  |  |  |  |
| 21 | All external fittings are properly labeled |  |  |  |  |  |
|  | FINISHING SKILLS |  |  |  |  |  |
| 22 | Joints brushed for cleanliness |  |  |  |  |  |
| 23 | Finished project is comparable to those made in industry |  |  |  |  |  |
| 24 | Construction is free from errors |  |  |  |  |  |
| 25 | Good attitude exhibited during the project execution |  |  |  |  |  |
| 26 | Confidence exhibited during project execution |  |  |  |  |  |
| 27 | Project limitations are clearly explained |  |  |  |  |  |
|  | TESTING SKILLS |  |  |  |  |  |
| 28 | Right test instrument provided |  |  |  |  |  |
| 29 | Appropriate tests are carried out |  |  |  |  |  |
| 30 | Test results reflect results of the planning stage |  |  |  |  |  |
| 31 | All functions operated well |  |  |  |  |  |
| 32 | No open circuit identified |  |  |  |  |  |
| 33 | No short circuit identified |  |  |  |  |  |
| 34 | Project operated over a long period of time |  |  |  |  |  |
| 35 | Project was executed safely |  |  |  |  |  |