# The Correlation between Road Network Performance and Land Price: Case Study Salatiga City

Title in English

Edwin Hidayat, Iwan Rudiarto, Walter de Vries

Received: --/--/----		 Accepted: --/--/----

Abstract: Many aspects should be considered in planning a sustainable city, two of them are transportation planning and population growth. These aspects have an essential role in changing the urban structure and the occupancy rate of a city. Population growth always related to people activity, particularly social and economic activities whereas road is the primary transportation to support people activities. Moreover, an increasing population means increasing the need for land for housing purpose. This need automatically triggered the land price fluctuation. This paper aims to examine the correlation of road network performance which represented by accessibility and mobility toward land price. The first method is secondary data collection such as the road network map, land price, and demographic data. Secondly, measure the road length using a GIS-based approach. Subsequently, statistical analysis is applied to understand the correlation among those data. The results showed that accessibility and mobility give positive relationship to the land price. However, in term of influence level, accessibility has a weak influence on the land price. Mobility has a relatively significant influence on land price.

Keywords: Road Network, Mobility, Accessibility, Land Price

# INTRODUCTION

In general, a city is a place of opportunity compared to a village. There are two phenomena which represent this statement. On the one hand, the migration from rural to urban is linked with the economic aspect and the opportunity to get a better life (Tacoli, McGranahan, & Satterthwaite, 2015 ) and (Yusoff, 216). As a result, many people move to the urban area as known as urbanization. In Indonesia, rural-urban migration is a dominant factor that causes high population growth in urban areas (Noveria, 2010).

On the second hand, there is a phenomenon called urban agglomeration which can be identified by commute trip (Wijaya, Kurniawati, & Hutama, 2018) and (Liu, 2018). The definition of urban agglomeration is numerous and depend on the user perspective. In term of functional interconnectivity, agglomeration is mainly deﬁned as an “urban ﬁeld” and “urban functional economic zones” concepts. People commute from the peripheral regions to the city center whereas the maximum reachable areas should be within daily commutes (Fanga & Yu, 2017). Urban field is a term for the place to live/housing area, and urban functional economic zones is a term for a place to works whereas road network is the infrastructure to support the commute trip.

Increasing population regarding the urbanization and expanding urban field related to agglomeration is equal to increasing the demand for the place to live, either housing or land for residential. Population growth led to the expansion of regional functions and increased demand for uncontrolled housing (Lestari & Djumiko, 2017) and (Bhatta, 2010). Meanwhile, uncontrolled/sporadic development of land for housing resulted in fluctuation of land price, particularly which related to green area and public facilities (Han, Chen, Shu, Lin, & Qiu, 2017) and (Bo, Zhicui, & Ning, 2015). This fluctuation can be predicted by using some approaches, the factor of property investment has been modeled (Sampathkumara, Santhi, & Vanjinathan, 2015), the mobility and utility parameters (Hidayat, Rudiarto, Siegert, & Vries, 2018) and the approach of using GIS-based model  (Yang, et al., 2015).

Development of housing area needs several basic facilities to support their daily needs such as the marketplace, education facility, and health facility whereas road network has a vital role in connecting those facilities (Levashev, 2017) and (Giuffrida, Ignaccoloa, Inturria, Rofè, & Calabrò, 2017). In addition, the road network has a function in facilitating the people activities, and a good road infrastructure gives a positive trend in support the economic growth (Ivanova & Masarova, 2013) and (Meersman & Nazemzadeh, 2017).

In Indonesia, road network plays a dominant role in transportation. Accessibility and mobility are the approaches which can represent the road network in facilitating the people activities. Accessibility refers to the ease of reaching goods, services, events, and destinations. While, mobility refers to physical movement, measured by trips, distance, and speed (Litman, 2017); and (Rau & Vega, 2012). A similar statement about accessibility, it is a measurement for road users to reach an activity center within a road-serviced area. While mobility is a measurement of road service as measured by the convenience of each people in traveling through the road to achieve its objectives (Iskandar, 2011).

Eventually, urbanization and agglomeration are created a high demand for land, and uncontrolled settlement area. In consequences, this demand has triggered a fluctuation in land price. Moreover, the sporadic development of housing land gives impact on the low road-service. Road network cannot cover all housing area due to uncontrolled city development. Thus, this paper aims to conceive the correlation between road network service which represent accessibility and mobility and the un-well-planned housing which described by the land price.

# Road Network Performance

Previous studies have shown many indicators to measure road network performance. Each indicator is developed depending on the perspective that represents the functionality of a road network from a specific aspect. From a traffic engineering perspective, road performance based on the quality of traffic flow, geometric characteristics, and delay. Others address road performance from road safety point of view, such as the number of traffic accidents. The impact of road activity, air pollution, and noise, for instance, are proposed as indicators for measure the performance of the road environment. Another indicator lies in the function of accessibility for the ease to reach destinations (TAC, 2006); (Fancello, Carta, & Fadda, 2014); (Qin, He, & Ni, 2014). In this study, the perspective of easiness to reach the destination is selected.

The road network performance is calculated from the people activity perspective. Accessibility and mobility are the key indicators which have a connection with the land price functionality. There are many definitions of accessibility in the literature. However, most of the early studies are employed the function of location, distance and time spent (Balijepalli & Oppong, 2014) and (Ford, Barr, Dawson, & James, 2015). The accessibility is expressed as a delegation unit of road network based on (Oemar & Daud, 2013) as follows:

AC=RLAR

Whereas:

AC	= accessibility (km/km2)

RL	= road length (km)

AR	= area (km2)

In term of mobility, the chosen formula is a representative unit of people traveling through the road network. If the road network is poorly maintained, it will constrain mobility and the economic growth as well as interrupted social benefits (Burningham & Stankevich, 2005). Mobility is important for the individual trip because always influenced by population, either population size or population density (Pojani & Stead, 2015). Mobility is expressed according to (Oemar & Daud, 2013) as follows:

MO=RLPO

Whereas:

MO	= mobility (km/1000 persons)

RL	= road length (km)

PO	= population per 1000 persons (1000 persons)

# Statistical Analysis

In this case, the statistical analysis is employed to examine the correlation coefficient by using correlation analysis and influence level using regression analysis (linear and non-linear) among some variables, namely land price, accessibility, and mobility.

The correlation analysis is a term to denote the relationship between two (or more) quantitative variables. This analysis is fundamentally based on the assumption of a straight-line (linear) relationship between the quantitative variables (Gogtay & Thatte, 2017). Correlation coefficients do not give information about whether one variable moves in response to another. There is no attempt to establish the dependent variable and the other as an independent variable. The magnitude of the correlation coefficient can be known based on the dispersion of the meeting points between two variables such as X and Y. If the points are in one line, then the correlation coefficient = 1 or -1 meaning the correlation is strong. When the points form a circle, the correlation coefficient = 0 meaning the correlation is weak.

The Pearson coefficient or well knew with Product-Moment correlation is applied to this case. The definition of this analysis is the ratio of the covariance of two variables representing a set of numerical data, normalized to the square root of their variances (Hall, 2015). The formula for this functions as follows:

r=∑xy

Whereas:

r	= correlation between variable x dan y

x	= (xi – X)

y 	= (yi - Y)

Afterward, the correlation coefficient needs to be checked to understand the significance level by using t-test. The result of t-test is compared with the t-tabel. For multiple correlations, the formula as follows:

Ry.x1x2=r2yx1+r2yx2−2ryx1ryx2rx1x21−r2x1x2

Whereas :

Ry.x1x2	= multivariable correlation between variables x1 and x2 together with variable y

ryx1 	= Product-Moment correlation between x1 and y

ryx2 	= Product-Moment correlation between x2 and y

rx1x2 	= Product-Moment correlation between x1 and x2

The significance of multivariable correlation is checked by using f-test. Then, the f-test value is compared with the f-table.

The regression analysis is divided into linear and non-linear. Regression measures how much a variable affects other variables, so it can be used to forecast the value of a variable based on other variables. The variable to be define (land price) is called the dependent (response) variable, and variables that explain it (accessibility, mobility) are called independent variables or predictor variables. Regression analysis employs a model that describes the relationships between the dependent variables and the independent variables in a simplified mathematical form (Schneider, Hommel, & Blettner, 2010). The formula for a linear multivariable regression as follows:

Y=a+b1×X1+b2×X2+...+bn×Xn

Whereas:

Y	= dependent variable

Xi 	= independent variables

a 	= constant (y-intersect)

b1 	= regression coefficient of the variable X

Meanwhile, if the scatterplot of data does not follow a straight line, but follows a specific curve form. Then, a suitable analysis to explain the relationship between X and Y is a non-linear regression. The non-linear regression model is a generalization of the linear regression model in which the conditional mean of the response variable is not a linear function of the parameters (Fox & Weisberg, 2010). This analysis can give better accuracy than linear regression because when estimating the model used algorithm iteration. There are several non-linear regressions namely, polynomial, exponential, reciprocal and others.

The regression analysis is interpreted by looking at the coefficient of determination (R2). It reflects the fraction of variation in the Y-values that is explained by the regression line (Schneider, Hommel, & Blettner, 2010). R2 provides information about the strength and direction of a relationship between two continuous variables. If R2 = ± 1 means perfect linear and monotone relationship. The closer R2 to 1 or –1, the stronger the relationship. If R2 = 0 means no linear or monotone relationship.

# Overview of Road Network and Land Market in Salatiga

Salatiga city is divided into four districts and 23 subdistricts. Total population in 2016 is 186.420 people with the total area of 56.781 square kilometers (BPS, Salatiga Municipality in Figures, 2017). In general, the road network system in Indonesia is classified into 5 categories, namely, toll road or freeway, artery road for connecting between national activity center, collector road for connecting between districts (urban road), the local road for connecting between sub-districts, and neighborhood road usually in the settlement area (Indonesia, 2004). In Salatiga, the arterial road as much as 11.795 km, the collector road is 112.955 km, the local road is 210.250 km and around 80,000 km of the neighborhood roads. Regarding the pavement type, from 415,000 km in total length of roads in Salatiga, 307.965 km are asphalt roads, and 107.035 km is soil road (BPS, Salatiga City in Figure, 2016). Neighborhood roads mainly have non-asphalt pavement due to lower vehicle loads, the pavement usually compacted soil or concrete block. Figure 1 presents the road network in Salatiga City as well as the land price.

Source: (Hidayat E. , 2017)

Figure 1. Road Network and Land Price in Salatiga

The highest land price in Salatiga city is mostly located in the district of  Sidorejo which laid on the north-west of Salatiga. Regarding road infrastructure, this district has a good road network and traversed by the arterial road. Sidorejo district has many urban transport routes and a bus station which located in this district too. In general, districts with the lowest land price are Argomulyo and Sidomukti whereas their location is in the south-area of Salatiga city. The road network in these areas in sufficient which mostly comprises of the local road.

Meanwhile, the land transaction in Salatiga is driven for housing. The new development of housing triggers the need for land. The land transaction fluctuates every year along with the permit for construction of new buildings. The land market profile in 2016 as much as 291 building license is issued and 1506 land transaction (buy and sell). Moreover, the land tenure system in Indonesia consists of Hak Milik or the highest level equal with freehold tenure, Hak Guna Bangunan (HGB) means right for the building only, Hak Pakai or a right to use, and Hak Tanggungan or a guaranteed right, Wakaf means land grant for religious purposes. In sequence, the total issued in 2016 for each tenure are 2252, 0, 226, 1433 and 13 land titles (BPS, Salatiga Municipality in Figures, 2017).

# METHODOLOGY

This study is adopted a quantitative approach, and the methodology comprises several steps. The first step is to arrange the research design that related to the research aims and the background problem. The second step is secondary data collection which gathered from the local agencies. The road length data is generated from the GIS map came from the agency for planning and development (Bappeda), the land price data is gathered from the property agent in Salatiga whereas the price consists of the offer price from the seller and an additional fee for the agent, demographic data is collected from the central statistical bureau (BPS).

Data preparation is required in the analysis step. The road length consists of neighborhood road, local road, collector road, and the arterial road are resulted from the road network in the case study location by using a GIS-based approach. The land price in this analysis consists of 3 land prices per subdistrict, then to be average to represent the land price per subdistrict. Demographic data for measure the accessibility is total square area per subdistrict, while demographic data for mobility is the total population.

Subsequently, accessibility and mobility are measured by using the accessibility equation and mobility equation. Then, statistical analysis is employed to reveal the correlation coefficient. Then, linear and non-linear regression analysis is executed to understand the influence level. The last step is the interpretation and discussion of the analysis result.

# RESULT AND DISCUSSION

After following the methodology, the result of the correlation analysis can be seen in table 1 as follows:

Table 1. Result of Correlation Analysis

| No | Variables | Correlation Coefficient | t-test estimated | 95% t-test table |
| --- | --- | --- | --- | --- |
| 1 | Accessibility & Land Price | 0.81 | 6.32 | 2.83 |
| 2 | Mobility & Land Price | 0.69 | 4.32 | 2,83 |

Source: Analysis, 2018

Based on table 1, the correlation coefficients between accessibility and land price as well as between mobility and land price resulted in a strong correlation between those variables because the value is >0.6. The correlation coefficient also has a positive value, and it means much better the accessibility, the higher land price. It also has the same meaning for mobility and land price, much better the mobility, the higher land price. Subsequently, the correlation between those variables is significant based on the t-test estimated which has a higher value compared to t-test table.

Regarding multivariable correlation analysis, the correlation coefficient for accessibility, mobility, and land price can be seen in table 2 as follows:

Table 2. Result of Multivariable Correlation Analysis

| No | Variables | Correlation Coefficient | f-test estimated | 95% f-test table |
| --- | --- | --- | --- | --- |
| 1 | Accessibility, Mobility, Land Price | 0.81 | 19.32 | 3.49 |

Source: Analysis, 2018

Table 2 presents the multivariable correlation among accessibility, mobility and land price. The results show that the correlation coefficient is strong because of 0.81>0.6. The result indicates that these three variables have a positive correlation. Then, comparison of f-test estimated and f-test table also showed a significant relationship among those variables because the f-test estimated is higher to the f-test table.

Moreover, the influence level of accessibility, mobility and land price is measured by using regression analysis, either linear or non-linear. The causality between variables of accessibility and land price can be seen in figure 2 as follows:

Source: Analysis, 2018

Figure 2. Regression Analysis For Accessibility and Land Price

Figure 2 depicts information about the effect of accessibility on land price. This graph shows dotted-line which represent the linear regression and stripped-line which represent the non-linear regression (polynomial). Both lines resulted in a small number of R2 or weak relationship because the value is <0.5. It means the accessibility cannot be used for predicting the land price due to small response between the independent and dependent variable.

The cause-and-effect relationship between mobility and land price can be seen in figure 3 as follow:

Source: Analysis, 2018

Figure 3. Regression Analysis For Mobility and Land Price

Based on figure 3, it can be seen that the line-graph portrays the causality relationship between mobility and land price. However, the coefficient determination or R2 has resulted in insignificant value for linear regression analysis which is 0.48<0.5. The non-linear regression used here is a polynomial and generated a relatively better R2  value, namely 0.54>0.5. This value means as much as 54% of the land price is influenced by mobility in the case study location. Thus, there are 46% is influenced by other factors.

# CONCLUSION

Elaboration of the linkages between road network performance and land price is examined using statistical analysis. The analysis consists of two approaches, namely correlation and regression. The road network performances are accessibility and mobility. The land price is a dynamic effect of sporadic/uncontrolled housing development.

The correlation between accessibility and land price have resulted in a positive and strong correlation coefficient. The comparison of t-test estimated and t-test table has revealed that this correlation was also significant. While the correlation between mobility and land price has also resulted in a positive and robust correlation coefficient too, the t-test estimated and t-test table has been compared and indicated a significant correlation. In addition, the multivariable correlation has been executed and resulting in a positive and strong correlation among accessibility, mobility and land price. The comparison of f-test estimated and f-test table is indicated a significant variable. Overall, the quantitative data of accessibility, mobility, and land price are produced a fundamental connection between those parameters.

The influence level of accessibility, mobility, and land price is determined using regression analysis. This analysis consists of linear regression and non-linear regression. The causal effect of accessibility on land price has a weak influence for both linear and non-linear analysis. The accessibility cannot be used for forecasting the land price due to small response between those variables. A much better result has been showed by the cause-and-effect relationship between mobility and land price. The linear regression for mobility and land price have a higher value compare to accessibility and land price. Moreover, the non-linear regression (polynomial) has generated a relatively better coefficient determination. It is denoted that the land price is influenced by mobility, but this influence was not working for accessibility.

Finally, the road network performance can be concluded it has a strong correlation with the land price. However, the road network performance is not good enough for forecasting the land price due to small response between those variables. As a collateral effect, the road network performance has a limited influence towards uncontrolled housing development.

ACKNOWLEDGMENT

This research is supported by the Ministry of Public Works and Housing – Indonesia as a part of the scholarship for an M.Sc program at Technische Universitat Munchen.

# REFERENCES

Balijepalli, C., & Oppong, O. (2014). Measuring vulnerability of road network considering the extent of serviceability of critical road links in urban areas . Journal of Transport Geography, 39, 145-155.

Bhatta, B. (2010). Analysis of Urban Growth and Sprawl from Remote Sensing Data. Advances in Geographic Information Science.

Bo, X., Zhicui, L., & Ning, X. (2015). The Interactive Influence of Urbanization, land Price and Industrial Structure Adjustment - the Empirical Analysis from China. Journal of Business & Economic Policy Vol. 2, No. 3 , 81-91.

BPS. (2016). Salatiga City in Figure. Salatiga: Badan Pusat Statistik Kota Salatiga.

BPS. (2017). Salatiga Municipality in Figures. Kota Salatiga: Badan Pusat Statistik .

Burningham, S., & Stankevich, N. (2005). Why road maintenance is important and how to get it done . WASHINGTON, DC : THE WORLD BANK.

Fancello, G., Carta, M., & Fadda, P. (2014). A modeling tool for measuring the performance of urban road networks. Procedia - Social and Behavioral Sciences 111 .

Fanga, C., & Yu, D. (2017). Urban agglomeration: An evolving concept of an emerging phenomenon. Landscape and Urban Planning 162 , 126–136.

Ford, A. C., Barr, S. L., Dawson, R. J., & James, P. (2015). Transport Accessibility Analysis Using GIS: Assessing Sustainable Transport in London. ISPRS Int. Journal Geo-Information 4 , 124-149.

Fox, J., & Weisberg, S. (2010). Nonlinear Regression and Nonlinear Least Squares in R. In J. Fox, & S. Weisberg, An R Companion to Applied Regression, Second Edition. Sage Publications.

Giuffrida, N., Ignaccoloa, M., Inturria, G., Rofè, Y., & Calabrò, G. (2017). Investigating the Correlation between Transportation Social Need and Accessibility: the Case of Catania. Transportation Research Procedia 27 , 816–823.

Gogtay, N., & Thatte, U. (2017). Principles of Correlation Analysis. Journal of The Association of Physicians of India, Vol. 65, March .

Hall, G. (2015). Pearson’s correlation coefficient. London: Imperial College London.

Han, H., C. H., Shu, X., Lin, L., & Qiu, D. (2017). The Effects of Greenbelt Policies on Land Development: Evidence from the Deregulation of the Greenbelt in the Seoul Metropolitan Area. Sustainability MDPI, 9.

Hidayat, E. (2017). Modelling the Dynamics Interrelation between Mobility, Utility and Land Asking Price. Munchen: Master Thesis, Department of Civil, Geo and Environmental Engineering, Technische Universitat Munchen-Germany.

Hidayat, E., Rudiarto, I., Siegert, F., & Vries, W. D. (2018). Modeling the Dynamic Interrelations between Mobility, Utility, and Land Asking Price. IOP Conf. Series: Earth and Environmental Science 123 (2018). IOP Publishing.

Indonesia. (2004). Indonesian Law number 38 year 2004 about road. Jakarta: Ministry of State Secretariate.

Iskandar, H. (2011). Minimum Service Standard Analysis for Non Toll Roads. Jalan Jembatan Journal Vol 28, No 1 .

Ivanova, E., & Masarova, J. (2013). Importance Of Road Infrastructure In The Economic Development And Competitiveness. ECONOMICS AND MANAGEMENT 18 (2).

Lestari, D. S., & Djumiko. (2017). Perkembangan Perumahan Dan Permukiman Sebagai Penentu Arah Dan Bentuk Kebutuhan Permukiman Di Pinggiran Kota. e-Jurnal Teknik Sipil Dan Arsitektur Fakultas Teknik UTP Surakarta Vol. 21 No 25, Juli .

Levashev, A. (2017). Application of Geoinformation Technologies for the Transportation Demand Estimation. Transportation Research Procedia 20 , 406 – 411 .

Litman, T. (2017). Evaluating Accessibility for Transport Planning. Victoria Transport Policy Institute.

Liu, K. (2018). Spatiotemporal Analysis of Human Mobility in Manila Metropolitan Area with Person-Trip Data. Urban Sci. 2, 3.

Meersman, H., & Nazemzadeh, M. (2017). The contribution of transport infrastructure to economic activity: The case of Belgium. Case Studies on Transport Policy Volume 5, Issue 2, June , 316-324.

Noveria, M. (2010). Fenomena Urbanisasi Dan Kebijakan Penyediaan Perumahan Dan Permukiman Di Perkotaan Indonesia. Masyarakt Indonesia EDISI XXXVI / NO.2 / 2010 , 103-1024.

Oemar, & Daud, J. (2013). Study Of Road Network Performance In Regional District of Labuhan Batu. Jurnal Teknik Sipil Usu Vol 2, No 1 .

Pojani, D., & Stead, D. ( 2015). Sustainable Urban Transport in the Developing World: Beyond Megacities. Sustainability, 7.

Qin, J., He, Y., & Ni, L. (2014). Quantitative Efficiency Evaluation Method for Transportation Networks. Sustainability, 6.

Rau, H., & Vega, A. (2012). Spatial (Im)mobility and Accessibility in Ireland: Implications for Transport Policy. Growth And Change 43 (4), 667-697.

Sampathkumara, V., Santhi, H., & Vanjinathan, J. (2015). Forecasting the land price using statistical and neural network software. Procedia Computer Science 57 ( 2015 ) 112 – 121 , 112-121.

Schneider, A., Hommel, G., & Blettner, M. (2010). Linear Regression Analysis. Deutsches Ärzteblatt International 107 (44), 776-782.

TAC. (2006). Performance Measures for Road Networks: A Survey of Canadian Use . Transport Canada .

Tacoli, C., McGranahan, G., & Satterthwaite, D. (2015 ). Urbanization, Rural–urban Migration and Urban Poverty. London: International Organization for Migration (IOM).

Wijaya, H. B., Kurniawati, H., & Hutama, S. (2018). Industrialization Impact on Worker Mobility and Land Use in Peri Urban Area (Case study of Semarang District, Indonesia). IOP Conf. Series: Earth and Environmental Science 123. Solo, Central Java.

Yang, Y., Sun, Y., Li, S., S. Z., Wang, K., Hou, H., & Xu, S. (2015). A GIS-Based Web Approach for Serving Land Price Information. International Journal of Geo-Information 4, 2078-2093.

Yusoff, N. B. (216). Urbanization In Sustainable Economics Growth Of Peninsular Malaysia Northern Economic Corridor. South East Asia Journal of Contemporary Business, Economics and Law, Vol. 10, Issue 3 .

Institute of Road Engineering, Ministry of Public Works and Housing, Indonesia

Department of Urban and Regional Planning, Diponegoro University, Indonesia

Department of Civil, Geo and Environmental Engineering, Technische Universitat Munchen, Germany

Correspondence: edwin.hidayat@pusjatan.pu.go.id